Title: Scale-Dependent Controls on Plume Dispersion in Stable Boundary Layers over Shallow Terrain

Author: Benjamin Marosites

Paragraph 1: Despite their relative frequency, Stable Boundary Layers (SBLs) pose ongoing challenges for understanding atmospheric motion and transport. Atmospheric dispersion is well understood in fundamental principles; however, under calm, nocturnal conditions or when warm air moves over a cooler surface, air can become stably stratified. Under these stable regimes, turbulence is suppressed, and local-scale motions and terrain-generated flow patterns become more prominent, often resulting in irregular, unexpected plume evolution. Coupled with challenges to observe and simulate SBLs, research has lagged behind that of convective conditions (LeMone et al. 2019). These limitations severely undermine model confidence and pose broad, significant challenges across atmospheric and climate sciences, public health, and effective land management practices, such as the application of plant-protective products (PPP).

Paragraph 2: Using scanning aerosol backscatter lidar and high-frequency (20 Hz) wind data from the 2018 Stable Air Variability ANd Transport (SAVANT) campaign, this research will explore the roles that turbulence and submesoscale and larger motions play in plume spread and density under stable conditions. Following the methods of Hiscox et al. (2006) and Strawbridge (2006), the plume’s edges, spatial spread, and internal relative concentration distribution are determined. Metrics from the plume description will be compiled into a time series. Using a multiresolution decomposition (MRD), as proposed by Vickers and Mahrt (2003), wind data and plume description data will be decomposed to represent fluxes occurring on different averaging time scales. Existing plume theory suggests that larger, lower-frequency submesoscale motions should influence plume spread, affecting the plume's overall size and position, while higher-frequency turbulence should influence the internal density concentrations. These findings will provide a framework for improving parameterization of SBL dispersion in mesoscale and climate models.

Bibliography

Hiscox, April L., Carmen J. Nappo, and David R. Miller. 2006. “On the Use of Lidar Images of Smoke Plumes to Measure Dispersion Parameters in the Stable Boundary Layer.” Journal of Atmospheric and Oceanic Technology 23 (8): 1150–54.

LeMone, Margaret A., Wayne M. Angevine, Christopher S. Bretherton, Fei Chen, Jimy Dudhia, Evgeni Fedorovich, Kristina B. Katsaros, et al. 2019. “100 Years of Progress in Boundary Layer Meteorology.” Meteorological Monographs 59 (AMSMOGRAPHS-D-18-0013.1): 9.1-9.85.

Strawbridge, K. 2006. “Scanning Lidar: A Means of Characterizing the Noranda-Hornesmelter Plume.” Geochemistry: Exploration, Environment, Analysis 6 (August): 121–29.

Vickers, Dean, and L. Mahrt. 2003. “The Cospectral Gap and Turbulent Flux Calculations.” Journal of Atmospheric and Oceanic Technology 20 (5): 660–72.